Dr.YSR HORTICULTURAL UNIVERSITY

Dr.YSRHU - HORTICULTURAL RESEARCH STATION - MAHANANDI

☏: 7382633690, 9885522496  	    : hrs-mndi@drysrhu.edu.in

----------------------------------------------------------------------------------------------------------------------------------------

PURCHASE OF TRINOCULAR STEREO RESEARCH MICROSCOPE AND RELATED ACCESSORIES

Microscope Body: Apochromatic Greenough System. Parfocal corrected optics

| Illumination | LED Ring Illuminator, segmentable - 8 High Power LEDs Correlated color temperature ca. 5800 K, lifetime ca. 25.000 , - Encoder 1 for setting of segment modes, turning in 45° steps and auto rotation - Encoder 2 for dimming, storage and recall of one illumination setting - Separate On/Off switch - Segment modes: full, half, quarter circle, two opposite quarter circles. Dual spotlight guide with Cold-light - up to 600 lm light flux ca. 6200K color temperature - min. 50,000 operating hours and 6 memory cells by rotary push knob. |
| --- | --- |
| Zoom | Manually Operable Zoom 8:1 Switchable zoom clickstops - 10 click stop positions - (End Stop 0.63×), 0.65x, 0.8×, 1×, 1.25×, 1.6×, 2×, 2.5×, 3.2×, 4×, 5× Magnification Range without interchangeable optics: 6.3x- 50x with working distance : 92 mm Magnification Range with interchangeable optics – Min Magnification : 1.9 mm and Max Magnification – 100x. Vario front lens system 0.3x-0.5x FWD 233...90mm, 1.0X , 2.0x front optics must be provided. Trinocular tube - Eyepieces: 10x/ FOV 23 Viewing Angle: 35 deg. Interpupillary Adjustment: 55-75 mm |
| Stand | Stand base D310xW200xH90 mm - Working surface D195xW160 mm - Interfaces d=84 mm for stages and d=45 mm suitable for TL polarizer - Column 250 mm with drive and handle, lifting range 145 mm - mount d=76 mm, load capacity 5 Kg, adjustable friction - Built-in LED transillumination unit with rotatable and slidable mirror for brightfield, darkfield and oblique transillumination - 2x sockets for IVI and illuminator - Separate RL/TL controls for on/off/dimming - Integrated power unit 12V DC 24W/100...240V AC/50...60Hz Additionally included: - Glass plate D=84x5 mm - Country-specific power cable EURO C8 - Dust cover |
| Camera | 4k resolution camera with minimum 12 mega pixels with CMOS sensor with rolling shutter, Pixel size 1.8µm x 1.8µm or higher, Digitization 3 × 8 bit/ pixel Exposure Time Range (Integration time) 0.1 ms – 1 s Gain 0× – 27× adjustable ,Image enhancement functions Active denoising, active sharpening, auto white balance Automatic features Automatic exposure and gain regulation at Ultra HD resolution (4K), fast live image under low light conditions, on-screen display without any additional PC or software required where You can add measurements, markers or text annotations to an image in live view. Interface 1× HDMI for monitor 1× USB 3.0 Type-C for flash drive, Wi-Fi adapter or PC connection,2× USB 2.0 Type-A for mouse and keyboard,1× RJ45 (Ethernet) for LAN connection 1× M8 for power and communication with dedicated stands. " |
| Software | User interface with drivers for cameras, configurable graphical user interface including workbench concept - Image acquisition with b/w, rgb, high-resolution and high-sensitivity cameras - Panorama for acquisition of Tiles Images with manual stage - Online Panorama for creation of stitched images on un-coded/ un- motorized stages - Video recorder functionality with movie export functionality - Manual Extended Focus (EDF) capability - Image processing: standard operations for image optimization (contrast, brightness, gamma, colors, smoothing, sharpening, geometric corrections...) - Interactive/dimensional measurements and text annotations - Windows multi-user functionality |
| Computer | CPU Intel i5 or i7 or AMD Ryzen 5 or 7 Ryzen / OS Windows 11 / 16 GB/ 1TB SSD/ 27-inch FHD (1920 x 1080) |
|  | Microscope should be from same manufacturer. |
|  | Warranty 01 year from the date of installation. |

Dr. Y S R HORTICULTURAL UNIVERSITY

HORTICULTURAL RESEARCH STATION-MAHANANDI

NANDYAL DISTRICT-518502, A.P.

Tender Notice No			:  NIT. No:2/ Dr.YSRHU/HRS-MND/2025-26

Date of Commencement of	: 12.03.2026

Downloading of the

bid document

Last date for receipt of bids. 	: 20.03.2026 AT 04.00PM

TIME AND DATE OF

Presentation of  Detailed Project Report	: 21.03.2026 AT 10.30 AM

and opening of Technical and Price bid  at

Dr Y S R HORTICULTURAL UNIVERSITY

HORTICULTURAL RESEARCH STATION - 518502

MAHANANDI, NANDYAL (Dt.)

ADDRESS FOR COMMUNICATION:

THE SENIOR SCIENTIST (PL.PATH) & HEAD

HORTICULTURAL RESEARCH STATION MAHANANDI,

MC FARM-518502

NANDYAL DISTRICT, ANDHRA PRADESH

Cont. No: 7382633690, 9885522496

Email: hrs-mndi@drysrhu.edu.in

The tender document containing terms and conditions for the execution of this project along with specifications and EMD to be paid are appended.

TERMS AND CONDITIONS

  - The item is to be installed at Horticultural Research Station-MAHANANDI, Nandyal District of Andhra Pradesh.

  - The bidder is required to furnish their previous experience certificate along with details of previously supplied with that of similar specifications.

  - The supply of equipments must be carried out as per the standard specifications under the supervision of concerned authority of Horticultural Research Station and has to be got certified by him, duly handing over all warranty certificates for the structure and electrical accessories.

  - All taxes and duties to be paid to the government needs to be included in price quoted in all the bills.

V)	The bids should be submitted with in time specified at the office of Senior Scientist (Pl.Path) & Head, Horticultural Research Station, Mahanandi along with EMD as specified here under against each item.

| Sl.No | Equipment | EMD Amount to be paid (in Rupees) |
| --- | --- | --- |
| 1 | Trinocular Stereo Research Microscope and related Accessories | 5000/- |

Bidders must pay the EMD amount specified as above against each of the item in the form of crossed D.D. drawn in favor of The Comptroller,              Dr YSRHU, Venkataramannagudem, Tadepalligudem payable at Union Bank/ any nationalized bank,  Tadepalligudem. Tender documents without earnest money deposit will be rejected and processing fee of Rs.1,000/- (Rupees One thousand only) through DD in favor of The Comptroller, Dr YSRHU, Venkataramannagudem, Tadepalligudem payable at Andhra Bank/ any nationalized bank,  Tadepalligudem which is non-refundable.

VI)   Specifications given are indicative and the tender is to give a detailed description and specifications on each section. The same would be given weightage in finalizing the tender. Rate of the each equipment mentioned should be quoted compulsorily.

VII)     The material/equipments to be supplied by the tenderers should be branded and meet the quality standard and as per laws Norms. Period of warranty shall be 1 year from the date of supply.

VIII)   The specifications and other conditions prescribed in Annexure – A of this

Tender shall also be treated as part of these tender documents for all purposes.

  - Terms of  Payment:

| 1) Upon supply & erection of the total equipment and after successfully trial run of the unit. | 100% of the project cost. |
| --- | --- |

ELIGIBILITY CRITERIA:

- The preference would be given to constructor/manufacturer with experience of having under taken complete project on Turn-key Basis and having executed at similar projects in reputed Horticultural  or Agricultural Universities / State department of Horticulture / ICAR  institutes/private organizations or similar Government aided University or College in India or Abroad

- The Tenderer should submit certificate from the project client for award of contract and stage of satisfactory project completion (in terms of value) in original or its notarized copy for each project claimed to be submitted.

- The company bidding should be in the industry for at least 10 years in the same filed. Tenderer should have registered in SSI / Directorate of Industries as a Manufacturer or authorized dealer of a reputed company (Please attach certificate).

- Audited financial statements duly certified by chartered accountant for the last five financial years ending August, 31st, 2025.

- The buyer reserves the right to evaluate the tender on Technical Presentation/ capabilities and is not bound to accept lowest tender / quotations.

Evaluation of Bid:

- The Technical Bid Documents will be opened first and evaluated by the Tender Committee. Financial Bid Documents of only those bidders will be opened who have qualified in Technical Bid.

- The Competent Authority reserves the right to accept or reject any tender without any reason thereof.

- Prices to be quoted on FOR basis including all taxes including VAT/freight/ installation/commissioning/trial/training charges etc in Indian rupees.

- It is necessary to submit all the relevant documents like Pan Card, ISO Certification, VAT/Sales Tax Registration, SSI/ Manufacturing Registration, Certificate of Incorporation / Partnership Deed.

- Tenderer should submit the copies of Purchase Orders in support of their genuineness in supplying of the goods in reputed Universities / Colleges / Departments/institutes/private organizations. The tenderer should submit users list for the last 5 years including for term key basis projects involving erection of screen house and equipments.

The Dr YSRHU reserves the right to verify the claims made by the Bidder and to carry out the capacity assessment of the bidder and the Dr YSRHU decision shall be final in this regard.